[ECE-593: Fundamentals of Pre-Si Validation, Portland State University]

VERIFICATION PLAN

APB System Verification

| Document Information |  |
| --- | --- |
| Document ID | [APB-2026-001] |
| Project / Product | APB System Verification |
| Design Version | RTL v1.0 |
| Verification Level | Sub-System |
| Prepared By | Truong Le, Marco Martinez Corral (ECE 593 Spring 2026 Team 2) |
| Reviewed By | N/A |
| Approved By | N/A |
| Date Created | [2026-15-04] |
| Last Revised | [2026-20-04] |
| Revision | 0.1 - Draft |
| Status | [X ] Draft [ ] In Review [ ] Approved [ ] Superseded |
| Confidentiality | [X ] Public [ ] Internal [ ] Confidential [ ] Restricted |

Purpose Statement

This Verification Plan defines the strategy, environment, tools, and completion criteria for the functional verification of APB System Verification. It is structured in accordance with the hierarchical verification methodology described in Comprehensive Functional Verification by Bruce Wilde.

FOR ECE-593:  FUNDAMENTALS OF PRE-SI VALIDATION:  BY: VENKATESH PATIL

# Table of Contents

Table of Contents	2

1. Introduction	2

1.1 Purpose and Scope	3

1.2 Design Overview	3

1.3 Verification Hierarchy	3

1.4 References	3

1.5 Definitions, Acronyms, and Abbreviations	3

2. Functions to Be Verified	4

3. Functions Not Covered at This Verification Level	5

4. Verification Strategy and Methods	5

4.1 Overall Strategy	5

4.2 Testbench Architecture	5

4.2.1 Testbench Components	5

4.3 Stimulus Methods	6

4.3.1 Directed Tests	6

4.3.2 Constrained-Random Tests	6

4.3.3 Assertion-Based Verification	6

4.3.4 Formal Verification	6

4.4 Checking Strategy	6

5. Verification Environment	7

5.1 Simulation Environment	7

5.2 Coverage Plan	7

6. Required Tools	7

7. Completion Criteria	8

7.1 Simulation Completion Criteria	8

7.2 Formal Verification Completion Criteria	9

7.3 Sign-Off Checklist	9

8. Resources and Schedule	9

8.1 Human Resources	9

8.2 Hardware Resources	10

8.3 Schedule	10

9. Risk and Mitigation	10

10. Document Revision History	11

# 1. Introduction

## 1.1 Purpose and Scope

This Verification Plan (VP) describes the complete approach for functionally verifying APB System Verification. It establishes the verification scope, methods, environment, required tools, completion criteria, and resource plan.

This document applies to the following verification level:

- [ ] Block-level verification

- [x ] Sub-system verification

- [ ] Full-chip / Top-level verification

- [ ] System-level verification

## 1.2 Design Overview

The design under verification is an APB system consisting of 3 components: a controller and 2 peripherals. The function of this system is to establish the use of APB to communicate data (read/write) between the controller and a particular peripheral.

The design under verification (DUV) is a custom AMBA Advanced Peripheral Bus (APB) sub-system. It consists of 1 APB Master controller and 2 APB peripherals (Slaves): a standard 32-bit memory array (Slave 0 at base address 0x0000_0000) and a custom Timer module (Slave 1 at base address 0x0000_0080). The protocol operates with a 32-bit data bus and a 32-bit address bus. The system's primary function is to route read/write commands from the master to the targeted peripheral using the APB non-pipelined, two-phase (Setup and Access) protocol, properly handling wait states (pready).

## 1.3 Verification Hierarchy

Verification of this design follows a hierarchical approach consistent with Bruce Wilde's Comprehensive Functional Verification framework. Each level of the hierarchy addresses a specific set of functions and feeds results to the next level.

| Hierarchy Level | Design Scope | Covered In This VP | Referenced VP |
| --- | --- | --- | --- |
| Block | APB Master and Slaves (Memory/Timer) | [ ] Yes [X] No | [APB-MAS-2026-001], [APB-TP-2026-001], [APB-MP-2026-001] |
| Sub-system | APB System | [X] Yes [ ] No | [APB-2026-001] |
| Full-chip | N/A | [ ] Yes [x] No | N/A |
| System | N/A | [ ] Yes [x] No | N/A |

## 1.4 References

| Ref # | Document Title | Version | Date |
| --- | --- | --- | --- |
| [R01] | Design Specification – APB System Verification | [1.0] | [2026-04-19] |
| [R02] | Comprehensive Functional Verification – Bruce Wilde | 1st edition | 2005 |
| [R03] | AMBA APB Protocol Specification | [Issue E] | [2023-02-28] |

## 1.5 Definitions, Acronyms, and Abbreviations

| Term / Acronym | Definition |
| --- | --- |
| DUV | Design Under Verification |
| DUT | Device Under Test |
| UVM | Universal Verification Methodology |
| SV | SystemVerilog |
| TB | Testbench |
| FC | Functional Coverage |
| CC | Code Coverage |
| CR | Coverage Report |
| VP | Verification Plan |
| APB | Advanced Peripheral Bus |

# 2. Functions to Be Verified

This section lists all functions and features of the DUV that are within scope of this verification plan, as required by the verification hierarchy (Wilde, Chapters 3–4). Each function is assigned a unique identifier for traceability.

| FV-ID | Function / Feature Description | Design Ref | Priority | Verification Method |
| --- | --- | --- | --- | --- |
| FV-001 | Reset behavior – all registers reset to defined values | DS §2.1 | Critical | Directed + Functional Coverage |
| FV-002 | Correct setup for read/write cycles | DS §3.2 | Critical | Directed + Constrained-Random |
| FV-003 | Interface protocol compliance – APB3 | DS §4.0 | High | Protocol VIP + Constrained-Random |
| FV-004 | Data integrity – read/write correctness across all modes | DS §5.1 | High | Scoreboard-based checking |
| FV-005 | Interrupt handling – priority, masking, and clearing | DS §6.0 | High | Directed + Constrained-Random |
| FV-006 | Timer peripheral function - set/start/check status | DS §7.0 | High | Directed Sequences |
| FV-007 | Error Detection: Transaction Errors | DS §8.0 | High | Fault injection |
| FV-009 | Register map – read/write/reset value for all registers | DS §10 | Critical | Register Model (RAL) Automation |
| FV-010 | Read/Write with Waitstate | DS §11 | High | Directed Sequences |

Priority legend: Critical = must pass before tape-out; High = required for signoff; Medium = best-effort; Low = stretch goal.

# 3. Functions Not Covered at This Verification Level

The following functions are explicitly excluded from this plan because they will be verified at a different level of the verification hierarchy. A corresponding Verification Plan reference is provided for each item.

| NC-ID | Function / Feature | Rationale for Exclusion | Verified In (VP Reference) |
| --- | --- | --- | --- |
| NC-001 | AHB-to-APB bridge timing and arbitration | Requires full-chip context | [System VP – VP-SYS-001] |
| NC-002 | Multi-master arbitration and bus contention | Single-master configuration chosen; multi-master scenarios deferred | [Analog VP – VP-AMS-001] |
| NC-003 | Software driver interaction | Covered at system / emulation level | [System VP – VP-SYS-002] |
| NC-004 | Formal exhaustive proof of FSM state reachability | Out of project scope (Milestones 1 - 5) | N/A - Not planned |

# 4. Verification Strategy and Methods

Milestone 1 utilizes a traditional flat SystemVerilog testbench for basic interconnect routing sanity checks. Subsequent milestones will migrate to a UVM environment utilizing constrained-random stimulus, scoreboard-based checking, and functional coverage to achieve signoff (Milestone 5 and final deliverables).

## 4.1 Overall Strategy

The verification strategy follows a layered approach combining:

- Directed tests – to verify specific corner cases and compliance requirements.

- Constrained-random stimulus – to explore unanticipated scenarios and achieve broad coverage.

- Assertion-based verification (ABV) – to catch protocol violations and temporal properties at the point of occurrence.

- Functional coverage – to measure completeness of the constrained-random campaign.

- Code coverage – to identify unreached HDL constructs.

## 4.2 Testbench Architecture

Milestone 1 uses a traditional (non-class-based) flat SystemVerilog testbench. The top-level module APB_TB instantiates APB_SYS_DUT, drives clock and reset, and applies directed stimulus via task-based helper routines (resetDUT, doTransact). No UVM or interface objects are used at this milestone. Subsequent milestones will migrate to a class-based (MS2) and then UVM 1.2 (MS4) environment.

### 4.2.1 Testbench Components

| Component | Description | Reuse / New |
| --- | --- | --- |
| Top-level testbench | Instantiates DUV, connects clocks/resets and interfaces | New |
| UVM Test | Controls test scenario; extends uvm_test | New |
| UVM Env | Assembles agents, scoreboards, and coverage collectors | New |
| Agent(s) | One per interface; contains driver, monitor, sequencer | [ ] Reused [x] New |
| Scoreboard | Compares DUV output against reference model | [ ] Reused [x] New |
| Reference Model | Golden behavioral model of DUV | [ ] Reused [x] New |
| Coverage Collector | Implements functional coverage groups and bins | New |
| Assertion Library | SVA properties bound to DUV interfaces and internals | [ ] Reused [x] New |
| Register Model (RAL) | UVM register model generated from IP-XACT / RALF | [ ] Generated [x] Manual |

## 4.3 Stimulus Methods

### 4.3.1 Directed Tests

Milestone 1: Directed tests exercise fundamental APB read/write paths using the flat testbench task doTransact. Each test drives PADDR, PWDATA, and the R/W select signal, then checks that PRDATA and transfer-status outputs match expected values.

Directed testing will be used to check for various cases in the design. This testing will include the simple read/write test to different peripherals. For corner cases, this will check the boundary addresses of the memory peripheral and check the accessibility of registers in the timer peripheral.

- Reset and initialization sequences

- Sequential Reads/Writes

- Read/Write with Waitstates

- Error injection sequences (e.g., illegal transactions)

- Timer Setup/Start/Checking

### 4.3.2 Constrained-Random Tests

Constrained-random stimulus is planned from Milestone 2 onward. In MS2, a class-based Generator will produce randomized APBTransaction objects constrained to legal address ranges and data values. Coverage feedback (MS3) will guide generation toward under-exercised bins.

- Random transaction ordering with protocol-legal constraints

- Randomized delay injection between transactions

- Weighted coverage-driven generation for under-covered bins

- Address randomization constrained to valid Slave 0 / Slave 1 address spaces

### 4.3.3 Assertion-Based Verification

Assertion-based verification is planned from Milestone 3 onward. Initial assertion targets are listed below; implementation will be added to the class-based TB.

- Interface protocol assertions (e.g., valid/ready handshake, request-grant timing)

- Data-path integrity assertions (e.g., no X propagation on outputs)

- Safety/liveness properties bound using bind constructs

- APB Bus idle assertion

### 4.3.4 Formal Verification

Formal property checking is not planned within the MS1–MS5 scope of this project. The entries below are retained for completeness per the template; they may be revisited for future courses.

- Control FSM state reachability and deadlock freedom

- FIFO empty/full flag correctness

- Reset completeness proof

## 4.4 Checking Strategy

Result checking will use the following methods:

- Scoreboard-based checking – DUV outputs are compared transaction-by-transaction against the reference model.

- In-line assertion checking – SVA monitors detect protocol violations immediately.

- End-of-simulation report – summary of all pass/fail checks, coverage, and assertion statistics.

# 5. Verification Environment

This section describes the simulation and verification environment, in accordance with Wilde Chapters 3, 4, and 7–12.

## 5.1 Simulation Environment

| Simulation Environment Details |  |
| --- | --- |
| Simulator | Synopsys VCS |
| Simulator Version | VCS 2023.03-SP2 (PSU ECE Linux Lab) |
| Verification Methodology | UVM 1.2 |
| HDL Language | SystemVerilog IEEE 1800-2017 |
| Assertion Language | SystemVerilog Assertions (SVA) |
| Compile/Sim Options | TBD |
| Seed Management | TBD |
| Regression Framework | Makefile |

## 5.2 Coverage Plan

[Describe the functional coverage strategy. Define coverage groups and bins for each function listed in Section 2. Reference Wilde Chapter 9.]

| FV-ID | Coverage Group Name | Bins / Cross Points | Target (%) | Tool |
| --- | --- | --- | --- | --- |
| FV-001 | cg_reset | All registers × reset state | 100% | VCS |
| FV-002 | cg_APB_phases | All phase transitions (IDLE→SETUP, SETUP→ACCESS, ACCESS→IDLE, ACCESS→SETUP) x direction (read/write) | 100% | VCS |
| FV-003 | cg_protocol | All transaction types × error / no-error | 95% | VCS |
| FV-004 | cg_data_integrity | All address × data patterns × operation type | 90% | VCS |
| [FV-x] | [Coverage group] | [Describe bins] | [%] | [Tool] |

# 6. Required Tools

This section lists all software tools required to support the described verification environment, consistent with Wilde Chapters 5–6. Items marked [Procure] may drive the software procurement team; items marked [Internal] require support from internal EDA/flow teams.

| Category | Tool / Package | Version | Vendor / Source | Status |
| --- | --- | --- | --- | --- |
| Simulator | VCS | 2023.12-SP2 | Synopsys | [X ] Licensed [ ] Procure |
| Formal Tool | N/A | [X.X] | N/A | [ ] Licensed [ ] Procure |
| Coverage Merge | VCS Coverage | [X.X] | Synopsys | [X ] Licensed [ ] Procure |
| VIP | N/A | [X.X] | N/A | [ ] Licensed [ ] Procure |
| Regression | [LSF / Slurm / Make-based] | [X.X] | [Internal EDA Flow Team] | [ ] Available [ ] Internal |
| Linting | [Spyglass / Questa Lint] | [X.X] | [Synopsys / Siemens] | [ ] Licensed [ ] Procure |
| Clock Domain Check | [Spyglass CDC / 0-In CDC] | [X.X] | [Synopsys / Mentor] | [ ] Licensed [ ] Procure |
| Version Control | Git | N/A | GitHub | [X] Available [ ] Internal |
| Bug Tracking | Git | N/A | GitHub | [X] Available [ ] Internal |
| Documentation | Google Docs | N/A | Google | [X] Available [ ] Internal |

# 7. Completion Criteria

Verification is declared complete when ALL of the following measurable criteria have been satisfied, in accordance with Wilde Chapter 13. Each criterion must be formally signed off by the Verification Lead and Design Owner.

## 7.1 Simulation Completion Criteria

| # | Criterion | Target Threshold | Achieved Value |
| --- | --- | --- | --- |
| 1 | All directed tests pass with zero unexpected failures | 100% pass rate | [] / [%] |
| 2 | Constrained-random regression passes across N seeds using a class-based testbench | N ≥ 50 seeds (per Milestone 2) | [ ] / [N] |
| 3 | All functional coverage groups close | ≥ [95]% | [ ] / [%] |
| 4 | Statement and branch code coverage achieved | ≥ [90]% | [ ] / [%] |
| 5 | Toggle coverage achieved on DUV outputs and interfaces | ≥ [85]% | [ ] / [%] |
| 6 | FSM state and transition coverage achieved | 100% reachable states | [ ] / [%] |
| 7 | All SVA assertions pass (zero assertion failures in final regression) | 0 failures | [ ] / [N] |
| 8 | All open bugs triaged; no Critical or High severity bugs outstanding | 0 Critical / High open | [ ] / [N] |
| 9 | Waived coverage items documented and approved | Approval on record | [ ] Done |

## 7.2 Formal Verification Completion Criteria (TBD)

| # | Criterion | Target | Status |
| --- | --- | --- | --- |
| 1 | All formal properties proven or vacuity-free | PROVEN / BOUNDED | [ ] Done |
| 2 | No unreachable properties (vacuity checks passed) | 0 vacuous passes | [ ] Done |
| 3 | All counterexamples analyzed and resolved | 0 open CEX | [ ] Done |

## 7.3 Sign-Off Checklist

| Sign-Off Item | Responsible Party | Date | Sign |
| --- | --- | --- | --- |
| Simulation completion criteria met (Section 7.1) | Verification Lead | [Date] | ___ |
| Formal completion criteria met (Section 7.2) | Verification Lead | [Date] | ___ |
| Coverage waivers approved | Design Owner | [Date] | ___ |
| All high-priority bugs closed or deferred | Design Owner | [Date] | ___ |
| Verification plan reviewed and up-to-date | Verification Lead | [Date] | ___ |
| Regression results archived | Verification Lead | [Date] | ___ |
| Final sign-off approved by Program Management | Program Manager | [Date] | ___ |

# 8. Resources and Schedule

This section defines the human, hardware, and software resources required, and provides schedule estimates to tie the verification plan to program management, as required by Wilde's framework.

## 8.1 Human Resources

| Role | Name / Team | Allocation (FTE) | Responsibilities |
| --- | --- | --- | --- |
| Verification Lead | Truong/Marco | [X.X] | Plan, coordination, sign-off, risk management |
| Verification Engineer(s) | Truong/Marco | [X.X] | Testbench development, test writing, debug |
| Design Owner / RTL Engineer | Truong | [X.X] | Design clarifications, bug disposition |
| EDA / Flow Support | Marco | [X.X] | Tool setup, regression infrastructure |
| Program Manager | Marco | [X.X] | Schedule tracking, resource allocation, status reporting |

## 8.2 Hardware Resources

| Resource | Specification | Quantity | Availability |
| --- | --- | --- | --- |
| Simulation Compute Servers | PDX ECE Linux Servers | N/A | [X ] Available [ ] Required |
| LSF / Job Scheduler Capacity | N/A | N/A | [x] Available [ ] Required |
| Formal Compute Servers | PDX ECE Linux Servers | N/A | [x] Available [ ] Required |
| Shared File Storage | PDX MCECS Storage | [X] TB | [x] Available [ ] Required |
| Emulation Platform | N/A | N/A | [x] N/A [ ] Required |

## 8.3 Schedule

| # | Milestone / Deliverable | Planned Start | Planned End | Owner | Status |
| --- | --- | --- | --- | --- | --- |
| M1 | Verification Plan approved, RTL sourced, flat testbench creation and basic design functionality tested | [04/06] | [04/20] | Truong/Marco | [x] Open |
| M2 | Class based testbench infrastructure complete, including generator, drivers, and other components. 20-50 randomized bursts of data verified. | [4/20] | [4/30] | Truong/Marco | [ ] Open |
| M3 | Finalize RTL, complete class-based verification, add code/function coverage reports, update verification plan | [4/20] | [4/30] | RTL-Eng | [ ] Open |
| M4 | Develop UVM testbench, define UVM architecture with updates to VP | [4/30] | [05/14] | V-Eng | [ ] Open |
| M5 | UVM Architecture, environment, and testbench completion. All testcases completed and reflected in coverage reports. Bug injection & verification scenarios implemented. Final deliverables compiled: documents, RTL, files, report, presentation. | [05/14] | [05/28] | V-Eng | [ ] Open |

# 9. Risk and Mitigation

| # | Risk Description | Likelihood | Impact | Mitigation Plan |
| --- | --- | --- | --- | --- |
| R1 | RTL instability delaying testbench development | High | High | Freeze RTL interface specification; develop TB against stable interface spec first. |
| R2 | Coverage closure not achieved within schedule | Medium | High | Add constrained-random coverage-driven tests; increase compute resources; waiver process for unreachable bins. |
| R3 | Tool license unavailability causing simulation bottleneck | Low | High | Licenses available through PSU; schedule off-peak batch regressions, use “backup” servers if required. |
| R4 | Key verification engineer departure / availability risk | Medium | Medium | Cross-train second engineer; maintain up-to-date documentation. |
| R5 | Formal verification timeout on complex properties | Medium | Medium | Apply bounded model checking; abstract sub-blocks; consult formal tool support. |
| R6 | Custom APB timer: blocked slave integration | Medium | Medium | Simplify design of timer, reduce complexity from parameterization. Worse-case: replace with another memory peripheral |

# 10. Document Revision History

| Revision | Date | Author | Description of Change | Approved By |
| --- | --- | --- | --- | --- |
| 0.1 | [2026-04-17] | [Le, Martinez Corral] | Initial draft for internal review | Venkatesh Patil |
| 0.2 | [YYYY-MM-DD] | [Name] | Incorporated review comments from V-Lead | — |
| 1.0 | [YYYY-MM-DD] | [Name] | Approved and baselined | [Approver] |
| 1.1 | [YYYY-MM-DD] | [Name] | [Description of change, e.g., Added Section 9] | [Approver] |

End of Verification Plan  |  [Project Name]  |  [Document ID]