# ECE 595 Final Project Design Specification - APB System Verification

Team 2 - Truong Le and Marco Martinez Corral - Version 1.0

### Description

Our project will utilize an open-source implementation of an AMBA APB system configured with our own APB peripheral. APB (Advanced Peripheral Bus) is used to connect low-speed peripherals to a faster bus protocol (usually an AHB-to-APB bridge is used to connect an AHB (Advanced High-performance Bus) system to an APB system). The protocol is non-pipelined and is used for its low-power advantage compared to the more advanced AMBA buses.

We plan to utilize tom-urkin’s APB implementation in their AMBA repository (https://github.com/tom-urkin/AMBA) as a basis for the APB system. The default configuration of the in-place APB DUT uses 3 APB controllers and 3 APB peripherals. To reduce the scope of the project, we will focus on a 1-controller system that will have 2 APB peripherals: one being the default memory peripheral provided by the repository, and the other being a custom APB timer peripheral.

### Details

The system will have the following components:

- 1 x APB Controller

- 1 x APB Memory Peripheral

- 1 x APB Timer

The system needs to have the following things verified for proper function:

- Read function (w/ and w/out waitstates)

- Write function (w/ and w/out waitstates)

- Error response for read and write transfers

- Peripheral selection

- For custom peripheral:

  - Configurable timer and relevant registers

  - Parameterized for clock division

  - Needs to work with current system